sudo emacs /etc/default/arduplane

TELEM1="-C /dev/ttyS1" TELEM2="-A udp:192.168.8.77:14550" #TELEM2="-C /dev/ttyAMA0" GPS="-B /dev/ttyS2"

# Options to pass to ArduPlane #ARDUPLANE_OPTS=$TELEM1 $TELEM2

# -A is a console switch (usually this is a Wi-Fi link) # -C is a telemetry switch # Usually this is either /dev/ttyAMA0 - UART connector on a Navio # or /dev/ttyUSB0 if you're using a serial to USB convertor

# -B or -E is used to specify non default GPS

sudo emacs /lib/systemd/system/arduplane.service

[Unit] Description=ArduPlane Service After=networking.service Conflicts=arducopter.service ardupilot.service ardurover.service

[Service] EnvironmentFile=/etc/default/arduplane ExecStartPre=/bin/bash -c "/bin/echo uart > /sys/devices/platform/ocp/ocp:P9_21_pinmux/state" ExecStartPre=/bin/bash -c "/bin/echo uart > /sys/devices/platform/ocp/ocp:P9_22_pinmux/state" ExecStartPre=/bin/bash -c "/bin/echo uart > /sys/devices/platform/ocp/ocp:P9_24_pinmux/state" ExecStartPre=/bin/bash -c "/bin/echo uart > /sys/devices/platform/ocp/ocp:P9_26_pinmux/state" ExecStartPre=/bin/bash -c "/bin/echo pruecapin_pu > /sys/devices/platform/ocp/ocp:P8_15_pinmux/state" ExecStart=/usr/bin/ardupilot/blue-arduplane $TELEM1 $TELEM2 $GPS

Restart=on-failure

[Install] WantedBy=multi-user.target